SMART IRRIGATION SYSTEM USING IOT, WEATHER DATA, AND AI

BY

SIVAPRADEEP M      (235113723)
                             SIVASAKTHI M         (235113724)  
                             SRIJAWHAR R           (235113725) 
                             SRIARIHARAN   N    ( 235113726)

Under the Guidance of

Dr .J .Vinci Shaalini M.Sc., B.Ed., Ph D., PGDCA

Assistant professor

PG & Research Department of Mathematics

Abstract

Introduction

Objectives

Literature Review (background on IoT, AI, smart irrigation, soil testing)

Ideology & Methodology

System Architecture (with block diagram/flow description)

Implementation Details (sensors, hardware, software)

Cost Estimation

Farmer Impact & Benefits

Sustainability & Scalability

Limitations & Challenges

Government Schemes & Support

Conclusion

References

ABSTRACT

Agriculture faces very severe challenges, i.e., inadequate water availability, soil erosion, and over-use of fertilizer and pesticides. These are issues of sustainability and productivity. Traditional irrigation is wasteful in terms of water and does not provide decent plant growth, and there is no inexpensive and available technology for farmers' use to assess soil health.

This dissertation presents Safe Crop, a novel solution in which smart irrigation based on IoT is complemented by Farm Trust soil and toxin monitoring, aided by climate data and artificial intelligence (AI). The system employs cheap sensors to keep a record of real-time soil moisture, temperature, pH, levels of nutrients, and toxins. The watering is automatically carried out, controlled by microcontrollers and by solenoid valves, which release water exactly when needed. Weather forecasts and inputs from sensors are processed by AI installations to anticipate watering requirements and optimize the use of resources. Safe Crop facilitates data-driven decisions for farmers and, simultaneously, promotes the environment by efficient use of water, reduction in chemicals, and improvement in soil.

INTRODUCTION

Agriculture is the foundation of human civilization and remains one of the most vital sectors of the global economy. However, modern farming faces significant challenges, including water scarcity, declining soil fertility, excessive pesticide use, and the increasing demand for sustainable food production. In countries like India, where agriculture is the primary livelihood for millions of farmers, these challenges are further intensified by unpredictable weather patterns, limited access to advanced technologies, and shrinking arable land.

Traditional irrigation practices often rely on guesswork, leading to either over-irrigation, which wastes water and damages crops, or under-irrigation, which reduces yields. Similarly, farmers frequently lack real-time insights into soil health, nutrient balance, and toxin levels, resulting in inefficient fertilizer use and higher risks of soil degradation and food contamination.

In response to these challenges, there is a need for responsive, data-driven agriculture solutions that are scalable, affordable, and easy to work with for farmers. With the growing availability of Internet of Things (IoT), Artificial Intelligence (AI), and weather-indexed analytics, a chance exists to transform agriculture. Soil moisture, pH, nutrition, and toxin levels could be automatically sensed by IoT sensors, and AI models could process this information along with forecasted weather patterns to make predictive choices about irrigation and soil amendments.

This dissertation presents Safe Crop, a novel combination of Smart Irrigation Systems and Farm Trust soil observation. Safe Crop uses IoT, AI, and climate information to enable farmers to better utilize water, observe soil conditions, and enhance crop yield. Through its emphasis on value for money, environmental sustainability, and empowerment, Safe Crop is poised to close the gap between state-of-the-art technology and ground-level agriculture, bringing precision farming to every type of farmer—from small holders to commercial farmers.

OBJECTIVES

The fundamental objective of this project is to develop and design a cheap, scalable, and smart agriculture system incorporating smart soil health management and smart irrigation in a combined manner. The specific objectives are: The project is to create a smart irrigation system based on IoT soil humidity, temperature, and moisture sensors for efficient irrigation management. It would incorporate Farm Trust soil measurement by adding pH, electrical conductivity (EC), and toxin detection sensors so as to assess soil quality in real time.For the integration of artificial intelligence and analysis of meteorological data for predictive irrigation schedules and soil health forecasts. Develop a platform targeting farmers with real-time alerts, actionable data, and local/offline capability where penetration of the internet is minimal.For scalability and affordability, designing the solution such that it is scalable both for small-scale farmers as well as large farms. In accordance with government initiatives and sustainable objectives, i.e., PMKSY (Pradhan Mantri Krishi Sinchayee Yojana), Atal Innovation Mission, and United Nations' Sustainable Development Goals (SDGs).

LITERATURE REVIEW

(Background of IoT, AI, smart irrigation, and soil testing for Safe Crop)

1. Internet of Things in Agriculture

IoT is sensor, actuator, and communication modules utilized for gathering and transferring data for control and monitoring. In farms, IoT is applied for real-time field monitoring (soil moisture, temperature, humidity, conductivity), remote control (valves, pumps), as well as gathering data for decision support. The most popular IoT architecture in agriculture is as follows: sensor nodes (low-power devices, e.g., ESP32/Arduino/LoRa nodes), gateways (LoRa WAN/GSM/Wi-Fi), cloud services (data storage, dashboards), and user interfaces (mobile apps, SMS).

Key advantages reported in the literature are higher resource efficiency (water, fertilizer), centralized, out-of-remote location monitoring of several sites, and time-series data supporting retrospective analysis. Key shortcomings reported in the literature are limitations in rural location connectivity, power availability to sensors, sensor drift and calibration requirements, and difficulty in supporting large sensor deployments. IoT prototypes in a low-cost form are ubiquitously available and a point of departure for most pilot projects, although rugged, reliable deployments for longer-term agriculture still demand thoughtful engineering and maintenance planning.

2. Farming With Artificial Intelligence (AI) and Machine Learning

Artificial intelligence (AI) and machine learning (ML) are utilized to derive actionable predictions and recommendations from raw IoT sensor streams and external information (rain forecasts, satellite imagery). In irrigation, popular AI applications are soil moisture time-series forecasting, evapotranspiration estimating, and predictive schedules that take into account plant water demand and rain forecasts. Supervised learning (regression, decision trees, random forest) and time-series models (LSTM, ARIMA) have been used to forecast moisture trends and when to irrigate. Reinforcement learning has been investigated for adaptive irrigation policies, where in repeated interactions, a model learns an optimal control policy.Anomaly detection (sensor malfunctions, disease/pest episodes induced by microclimate patterns), feature extraction from images (crop vigor, canopy coverage, pest/disease indicators) through computer vision, are assisted by AI. The shortcomings emphasized in the literature are a need for well-tagged data applied in training models, domain adaptation issues in porting models from regions/crops, explainability (farmers prefer transparent, trustworthy explanations for advice), and computational/energy constraints in supporting edge deployment. Cloud-edge hybrid models, or interpretable models, are suggested for deployments in real agriculture.

3. Intelligent Irrigation Methodologies and Algorithms

Smart irrigation encompasses hardware (smart valves, micro-sprinkler, drip system) and software (scheduling programs). Drip irrigation and fertigation are proven, water-saving methods in most crops; adding sensors and controls enables precision irrigation—water is used only where and when needed. Some of the popular algorithms used are: Threshold control: Simple current soil moisture-based if-then rules. Reactive and simple to apply. Model-based control: Uses soil-plant-atmosphere models and evapotranspiration calculations (Penman–Monteith, crop coefficients) to schedule irrigation.

optimization methods: Maximize product or minimize consumption of water subject to constraints (plant stress constraints, pump abilities). Machine learning forecasters: Predict soil moisture or water stress of crops, allowing for more adaptive schedules. Relative work has found that predictive and model-based methods reduce consumption of water and maintain or increase output compared to threshold-based control. Complexity of implementation and requirements for data, however, grow with algorithm sophistication. Operational applications often pair simple local reasoning at the device level with cloud prediction services at higher levels of scheduling.

4. Soil Testing, Nutrient Monitoring, and Toxin Screening Soil analysis has conventionally depended upon lab chemical assays for pH, Electrical conductivity (EC), organic matter, and macro/micronutrient content measurement (N, P, K, etc.). Laboratory tests are precise, but time-consuming and expensive for routine measurement. Rapid field tests—the colorimetric strips, handheld potentiometer meters (of pH), EC meters, and ion-selective electrodes—all collectively offer usable rough estimates in a very inexpensive manner. Colorimetric tests for pesticide residues are commercially bought in test strip form, though frequently sensitivity is poor and range narrow; confirmation in a lab is necessary for regulatory or export-grade quality.

New research investigates inexpensive sensor fusion (pH + EC + optical/colorimetric measurements + contextual IoT information) and machine or deep learning to predict wider soil health indicators from constrained sensor sets. There is strong interest in building rugged field-portable toxin detection (of generic families of pesticides) through biosensors, immunoassays, and electrochemical sensors. Difficulty in this regard is sensitivity, false alarms, cross-reactivity, shelf-life, and need for trained interpretation. Combining camera-based colorimetry (smart phone or inexpensive camera + ML) with strips is able to decrease reader bias and normalize results.

5. Traceability and Consumer-Facing Verification

The use of traceability systems (QR codes, blockchain-secured ledgers) is gaining popularity in providing provenance and certification of agricultural produce. The incorporation of sensor logs from farms, soil tests, and handling information into a consumer's accessible dashboard has the potential to command a price premium and satisfy export requirements. Literature identifies data integrity, users' privacy, and user-friendly interfaces for farmers and consumers as being significant. Traceability is efficient when complemented by third-party verification or trustworthy on-farm testing procedures.

6. Gaps, Opportunities, and Relevance to Safe Crop

Integration gap: Every solution addresses either precision irrigation, soil measurement, or traceability, not a composite set. Safe Crop’s simultaneous integration of irrigation control and field-level toxin/nutrient tracking and end-customer level traceability addresses such an integration gap. Affordability/cost: High-end commercial uses are typically out of smallholders' reach; cheap, modularized arrangements increase adoptability. Detection at the field level: Advanced, field-deployable, affordable detection of pesticide residues is where innovation is needed, most aggressively when coupled with smartphone-readable detection and ML for standardization. Implementation of edge and hybrid AI: These deployments would preferably be hybrid-local threshold control and cloud-centric AI for predictivity—to address heterogeneity in connectivity. Model compression and tiny enable constrained on-device inference where necessary. Users trust and explainability: Both farmers and consumers require transparent advice and tractability; UI/UX and explainable AI are necessary. Recommended Reading / Reference Types (to use when finalizing thesis) IoT in agriculture research review papers (journal proceedings and conference proceedings).Farm Beats, CropX, and other cases of scaling precision irrigation implementation. Journals for machine learning-oriented irrigation schedules and Tiny edge inferences. Field-portable detection of pesticides research (colorimetric strips, biosensors. Policy and programmed papers concerning PMKSY, Digital Agriculture initiatives, and procedures traceability.   

METHODLOGY

The Smart Irrigation using IoT Weather data  and Ai installation gathers clever watering systems and monitors the health of the soil, all from Farm Trust, with assistance from IoT devices, weather data, and AI predictions on what to do next. It is divided into four overall steps, you see. System Design first. Next, Data Collection. Then Processing and Analysis. And lastly, Deployment and Evaluation.

System Design.

They constructed the hardware and software in a modular fashion. That is, it fits for small test applications or large farm situations, essentially. Sensors that they utilize.Soil moisture sensors, capacitive type, to monitor water content in the soil.DHT11 or DHT22 for temperature and humidity measurements sensor to determine if the soil's too acidic or alkaline.EC sensor for electrical conductivity, which monitors nutrients essentially Colorimetric strips combined with an LDR or camera to detect pesticide residues or toxins.

Control components.

ESP32 or Arduino Node MCU serves as the brain, with Wi-Fi or GSM to communicate. Relay modules and solenoid valves control the irrigation pumps. The power source is batteries or solar installations.

Software side.

Firmware on Arduino IDE or MicroPython acquires data and executes the devices.An IoT dashboard such as Blynk or ThingsBoard, or even a bespoke app, displays live graphics.AI algorithms forecast when to water, based on training on soil and meteorological data sets.

Data Gathering.

Sensors capture information every few minutes or so. They record things like moisture in the soil, pH, EC, temperature, humidity, and if any toxins appear. Weather forecasts arrive via APIs, such as OpenWeatherMap or IMD. Data gets stored locally on SD cards or EEPROM. Occasionally, it syncs up to the cloud for longer views.

Processing and Analysis.

They employ basic threshold logic directly on the spot, such as local AI lite. If below thirty percent moisture, turn on irrigation. If greater than sixty percent, turn it off. For more intelligent forecasting, cloud or edge AI takes over. Machine learning learns irrigation requirements from moisture levels, evapotranspiration, and future weatherDecision trees or random forests are used in models to classify irrigate or not.LSTM is used for time-series prediction of soil moisture toxins and nutrients.AI camera image processing and ML detect color changes on strips. Quantifies amounts of chemicals as well.

Deployment and Evaluation.

They install a one-acre pilot. Two or three soil moisture sensors. One for pH. One for EC. And an irrigation control unitTry irrigation cycles with AI and without to check water savings.Compare soil toxin and nutrient data with lab tests for accuracyTake feedback from farmers regarding interface and price, to fine-tune.

Methodological Workflow.

Input layer. The soil and weather are directly measured by sensors.Processing layer. The microcontroller fetches data and tidies it up slightly.Decision layer. Straightforward rules combined with AI forecasts determine.Action layer. It waters automatically and provides soil warnings to farmers.Output layer. Alerts through dashboard, SMS, or app. QR codes for soil reports, to monitor all.

IDEALOGY

The fundamental philosophy of SafeCrop is to integrate affordability, sustainability, and ease of access in contemporary farming through the use of IoT and AI technologies. Agriculture forms the pillar of food security, but farmers encounter difficulties like volatile weather patterns, inefficient use of water, depletion of land, and concealed chemical residues. SafeCrop resolves such problems by following a farmer-centric strategy.

1. Farmer-Centric Innovation

Farmers themselves do not have access to modern technologies because of their cost and intricacy. Safe Crop provides affordable sensors, open-source hardware, and easy-to-use farmer apps to simplify technology and affordability.

The system operates in two modes:

Offline Mode (IoT-only): Threshold-based irrigation and soil health monitoring independent of the internet. Online Mode (IoT + AI): Sophisticated cloud/edge-based predictions for large farms and precision agriculture.

2. Sustainable Agriculture

Conventional irrigation techniques waste 40–50% of water. SafeCrop minimizes water usage by optimizing the irrigation cycle based on real-time weather and soil conditions. Soil degradation due to pesticides and fertilizers is monitored through toxin and nutrient tracking to provide healthy crops and safe food. Encourages sustainable farming by avoiding over-irrigation, over-fertilization, and chemical abuse.

3. Traceability & Food Safety

Buyers are increasingly demanding toxin-free food.SafeCrop provides electronic soil health reports, enabling buyers to be shown these by farmers, providing farm-to-market transparency. This enhances consumer confidence in farmers' produce, allowing premium price opportunities.

4. AI for Empowerment, Not Replacement

AI is not intended to replace farmers' decision-making, but to assist them with scientific understanding. Through the marriage of traditional farming practices with AI forecasting, Safe Crop enhances decision-making in irrigation and soil management

5. Inclusive Growth

The initiative foresees scaling from smallholders to agribusinesses.Farmers who lack connectivity can still gain benefits, while forward-thinking farmers with internet connectivity can unlock data-driven precision agriculture. Long-term vision: establishing a global Safe Crop farm network with mutualized data, enhancing farm AI for everyone.

SYSTEM ARCHITECTURE

Smart Irrigation system using IoT is a hybrid edge-and-cloud, modular IoT architecture that gathers field sensor data (microclimate, strip readers, soil), executes local control logic and edge inference, syncs to the cloud for long-term analytics and AI prediction, and offers farmer/consumer apps and traceability services (QR).

[Weather API]                [External Lab/API]

|                              |

v                              v

--------------------          --------------------

|   Cloud / Server  | <------> | 3rd Party Data  |

|  (Storage, AI,    |          | (market, export)|

|   Model Training) |          --------------------

--------------------

^        ^        \

|        |         \

MQTT/HTTPS  |     REST/Webhooks

|        |          \

-----------------------------         -----------------------

|   IoT Gateway / Edge      |  <----  | Farmer Mobile App   |

|  (Edge DB, TinyML, OTA)   |         | (Dashboard, Alerts) |

-----------------------------         -----------------------

^     ^     ^    ^   \

|     |     |    |    \

[Soil][pH/EC][Toxin][Temp][Camera]

Sensors Sensors Sensors  DHT  (Strip reader)

|     |     |    |

\     |     |    /

\    |     |   /

\   |     |  /

[Microcontroller Node (ESP32/LoRa Node)]

|

Relay / Valve

Pump

Components & Responsibilities Layer (Sensing & Actuation): Soil moisture sensors (capacitive)measure volumetric water content. pH sensor & EC probe — monitor acidity and conductivity (proxy for nutrient levels).

Toxin detection — colorimetric strip or handheld electrochemical/biosensors; optionally read by camera or LDR + ADC.

Temperature & Humidity sensor (DHT22 / SHT3x) — ambient microclimate. Camera / LDR module (optional) — standardize colorimetric strip reading using ML. Microcontroller: ESP32 / Arduino / LoRa node — aggregates sensors, executes local logic, drives relays/valves.Actuators: Solenoid valves, pump controllers, fertigation unit (relay + MOSFET drivers).Edge / Gateway Layer

Gateway device (Raspberry Pi / industrial gateway / LoRaWAN gateway): groups nodes, local preprocessing, temporary storage, TinyML inference (if model is small), and deals with intermittent connectivity.

Local database (SQLite) for buffering.

Edge services: OTA firmware updates, device provisioning, simple analytics, and local web UI for field technicians.

Cloud / Server Layer

Device Management (register devices, monitoring, health).

Time-series data store (InfluxDB/TimescaleDB) and raw data lake (S3/GCS).

AI/ML services:

Model training (historical data): regression/time-series/LSTM/RandomForest.

Prediction API: irrigation requirement, pest/disease threat, nutrient adequacy.Image processing pipeline for strip colorimetry (if utilized).

Application server / API: REST APIs for web/mobile applications, QR generation, integration with third-party systems (marketplaces, lab confirmations).

Traceability ledger: secure batch QR logs (may be simple DB + cryptographic hash or blockchain if needed).

User Interfaces

Farmer mobile app / SMS gateway: daily reminders, irrigation schedules, override, test strip logging, local language UI, offline capability.Consumer portal / QR viewer: scan QR → display batch water consumption, soil reports, toxin test status, harvest date.

Admin dashboard: farm clusters, FPO view, analytics, subsidy reporting.

Third-party & External IntegrationsWeather APIs (OpenWeatherMap / IMD data).Lab confirmation services (for regulatory validation).

Subsidy portals / FPO systems/market buyer APIs.

Data Flow (step-by-step)

Sensors take raw readings at configurable intervals (e.g., every 5–15 mins). Microcontroller preprocesses (smoothing, de-noise), stores locally, and transmits over MQTT/HTTP to the Gateway or directly to the cloud when connectivity is present. Gateway buffers data; executes TinyML model for real-time decisions (pump ON/OFF) if so configured.

Data transmitted to Cloud: stored in time-series DB; invokes server-side AI prediction engine.

AI generates irrigation recommendation/risk alerts and stores the decision in the cloud. App retrieves recommendation; cloud sends push/SMS; gateway executes actuation command, or farmer follows manual guidance. Soil toxin strip testing recorded (manually through app or auto-read through camera); confirmed entries create QR traceability for said harvest batch. Retrained models put to use through CI/CD to cloud/edge.

AI/ML Pipeline

Ingestion of data : time-series aggregation, feature engineering (soil moisture trend, estimate evapotranspiration, rolling averages).

Model training : supervised learning (regression for moisture prediction; classification for "Irrigate/Skip"); LSTM/GRU in case of plenty of data.Model assessment → RMSE, MAE, precision/recall (for classification); cross-validation between domains/crops.

Deployment :published models as REST microservices (cloud) and compressed variants (TensorFlow Lite / TinyML) deployed to gateway/edge for low-latency decision-making. Feedback loop → farmer confirmations and yield records utilized for model improvement.

Protocols & Standards

Device ⇌ Gateway: LoRaWAN (low-power, long-range), or Wi-Fi / ESP-HTTP / MQTT over TLS.Gateway ⇌ Cloud: MQTT over TLS or HTTPS REST.

APIs: RESTful APIs with OAuth2/JWT for auth.Data format: JSON / Protobuf for telemetry.ML: TensorFlow / PyTorch for training; TensorFlow Lite for edge inference.

Security & Privacy

Device authentication: each device uses TLS client certificates or per-device tokens.Encrypted transport: MQTT/TLS, HTTPS.Data encryption at rest: server-side encryption (S3/KMS).Access control: RBAC for farmer, FPO, and admin interfaces.Privacy: farmer data ownership policy; minimal personal data; opt-in for sharing traceability with buyers.

Audit logs: immutable record of test outcomes and trace events; optionally sign log entries cryptographically using hashing.

Fault Tolerance & Offline Design

Local buffering: microcontroller to sensor to gateway buffer on SD/SQLite.Graceful fallback: local control threshold-based if the cloud is not reachable.Watchdog & retries: automatic reconnection with exponential backoff.Edge inference: life-critical safety decisions (prevent floods) made locally.

Deployment & Maintenance Strategy

Pilot: 1-acre pilot with 3 moisture sensors, 1 pH, 1 EC, 1 actuator; test for 3 months over seasons.Scale: FPO model — central gateway per cluster, common weather station, per-acre sensors.Training: farmer onboarding, local technician training, and KVK collaboration.

Maintenance: routine calibration schedule, warranty replacement plan, SLA for gateway cloud connectivity.Firmware updates: secure OTA capability through the gateway.

Recommended Hardware & Software Stack (practical)

Field: ESP32 nodes, capacitive moisture sensors, Atlas Scientific pH/EC (or low-cost alternatives), DHT22/SHT31, 12V solenoid valve, 12V DC pump.Gateway: Raspberry Pi 4 / LoRaWAN gateway (The Things Network).Cloud: AWS IoT Core / Azure IoT / Google Cloud IoT; InfluxDB / TimescaleDB; S3-like storage.AI: Python (scikit-learn, TensorFlow, PyTorch); MLflow for tracking.Dashboard / App: React (web), React Native / Flutter (mobile), Blynk or ThingsBoard for a quick prototype.CI/CD: Docker, Kubernetes (for scale), GitHub Actions.

Scalability & Cost Considerations

Economies of scale: the cost of hardware per acre decreases when sensors are grouped in clusters and gateway/weather stations are shared. Cloud costs: ML training and time-series DB are major costs; utilize serverless workflows for pilots to minimize expense.Model complexity vs edge feasibility: begin with easy-to-predict models and gradually move to more complicated models to the cloud.

IMPLEMENTATION DETAILS

Hardware Implementation

Sensors

Soil Moisture (Capacitive): Two sensors per acre at varying depths (10–15 cm, 20–25 cm) to record real-time root zone data. pH & EC Probes: Pushed into the soil or blended soil-water slurry for calibration-based measurements. Toxin Detection Strips: Farmers immerse test strips in soil solution; an optional LDR + Camera module records strip color for automated digital logging. DHT22 Temperature & Humidity: Fixed inside a miniature weatherproof enclosure for microclimate monitoring.

Microcontroller

ESP32 Node MCU: Processes sensor inputs, preprocesses data, and communicates with the gateway/cloud. Attached to a 12V solenoid valve and relay module to switch irrigation pumps on/off. Power Supply12V DC battery or mini solar panel + charge controller to achieve field autonomy. Provides continuous operation in case of power outages.

2. Software Implementation

Firmware (ESP32/Arduino IDE) Samples sensor data every 5 minutes.

Utilizes median filtering to minimize sensor noise.Employed threshold logic (e.g., moisture level < 30% → irrigate ON).Sends JSON packets over the MQTT protocol to the IoT gateway. Gateway Software (Raspberry Pi / LoRaWAN) executes Node-RED / ThingsBoard Edge for routing data.

Buffers locally in SQLite when the cloud is not available.Executes TinyML inference (TensorFlow Lite models) for rapid irrigation decisions.Cloud Backend Database: InfluxDB / TimescaleDB holds time-series data.

API Layer: Python Flask / FastAPI manages communication between IoT devices and mobile app.AI Module: Regression models forecast irrigation requirements (dependent on moisture, evapotranspiration, forecast). Classification models identify soil nutrient shortage or toxin warnings. Computer Vision pipeline (OpenCV + TensorFlow Lite) identifies color intensity on strips for pesticide residue analysis.

3. Mobile App Implementation

Farmer Dashboard: Shows live soil moisture, pH, EC, and weather forecast.

Sends irrigation reminders: "Irrigate for 15 minutes today".Manual strip test entry or auto-upload of strip readings.Operates offline (stores and syncs data later).

Consumer/Buyer View: Adds QR code tied to farm information (soil health report, toxin report, harvest date).Develops transparency and trust for food safety.

4. Integration with AI

Offline Mode (IoT-only): Fixed thresholds are used (e.g., 25–35% moisture).

Online Mode (AI-powered): Utilizes historical soil data + weather API + evapotranspiration models to forecast irrigation timing.AI learns crop behavioral patterns seasonally (e.g., rice needs more standing water vs. tomato drip irrigation). Farmer confirmations (YES irrigated / NO skip) are utilized as feedback to enhance AI models.

5. Testing & Calibration

Moisture sensors referenced to gravimetric soil samples in the lab.

pH/EC probes calibrated with reference buffer solutions (pH 4, 7, 9; EC 1.4 mS/cm).Toxin strips tested with spiked sample soil-water and known pesticide/fertilizer concentrations.Field Trials: Demo plot 1-acre, halved into two halves:  Smart Irrigation vs traditional irrigation. Compare yield, water conserved, and soil nutrient condition.

6. Deployment Setup

Pilot Phase (1 acre)

2 × soil moisture sensors

1 × pH sensor

1 × EC sensor

1 × DHT22

1 × ESP32 microcontroller

1 × solenoid valve + pump control

Optional camera for strip test automationScaling

Phase (5–10 acres)Central gateway with LoRaWAN nodes lowers cost per acre. Common  weather station for precise AI forecasts.

7. Maintenance & Support

Sensors are calibrated every 3–6 months. App offers self-diagnostics (e.g., "Moisture sensor offline"). Local technicians are trained for battery replacement and sensor maintenance.

8. Results (Expected)

30–40% water savings using smart irrigation. Chemical use has been reduced due to nutrient/toxin monitoring.10–15% yield improvement (due to optimized water/nutrient balance).Market edge through traceable, safe, toxin-free produce.

COST ESTIMATION

The price of SafeCrop installation varies based on the deployment size (pilot vs. cluster-level) and if additional modules like AI strip readers or cloud integration are added. Following is a detailed list:

1. Pilot Project (1 Acre – Basic Setup)

Component\tQuantity\tUnit Cost (₹)\tTotal (₹)

Soil Moisture Sensor (Capacitive)\t2\t300\t600

Temperature & Humidity Sensor (DHT22)\t1\t250\t250

pH Sensor Kit\t1\t1,000\t1,000

EC Sensor\t1\t1,200\t1,200

ESP32 WiFi Microcontroller\t1\t800\t800

Relay Module\t1\t200\t200

Solenoid Valve (12V)\t1\t500\t500

Mini Water Pump (12V)\t1\t600\t600

Toxin Test Strips (pesticide/nitrate)\t10 pack\t500\t500

Camera Module (for strip reader – optional)	1	800	800

Wires, Breadboard, PCB, Enclosure, Power Supply	–	800	800

Total (Basic Setup)	–	–	₹7,250 – ₹8,250.

2. Advanced Features (Optional Add-ons)

Component	Quantity	Unit Cost (₹)	Total (₹)

LoRa Node + Gateway (for long-range farms)	1	5,000–8,000	5,000–8,000

Raspberry Pi Gateway (for local edge AI)	1	4,500	4,500

Solar Panel + Battery Pack	1	3,000	3,000

Cloud Hosting & AI Training (AWS/GCP/Azure – yearly)	–	~6,000–10,000	~6,000–10,000

3. Cost per Acre at Scale (5–10 Acres)

Shared Gateway & Weather Station reduces per-acre cost. Per-acre hardware cost comes down to ~₹5,000–₹6,000 when split among many farmers. Yearly maintenance (sensor calibration, replacements, data hosting) ~₹1,000 per acre.

4. Cost-Benefit Impact

Water Savings: 30–40% less water per acre → saves energy cost on pumping.

Yield Improvement: 10–15% increased yield due to balanced irrigation & nutrients.

Market Premium: 5–20% premium selling price for toxin-free, traceable produce.

ROI can be achieved within 1–2 crop cycles (6–12 months).

FAEMER IMPACT AND BENEFITS

Farmer Impact & Benefits

Smart Irrigation System Using IOT, weather data and AI is not only a technological proof of concept but also a farmer-focused intervention. The anticipated benefits are direct (short-term gains felt during the pilot) and indirect (long-term sustainability and livelihood benefits).

1. Water Savings

Farmers tend to irrigate on fixed timetables or according to individual judgment, which may result in over-irrigation or under-irrigation. With Smart AI-based decision support and soil moisture sensing, irrigation is used only when required. Anticipated water savings: between 30–40% per season over conventional practices. This directly saves electricity/diesel on pumping water costs.

2. Better Soil Health

Regular monitoring of the soil's pH and electrical conductivity (EC) prevents water/fertilizer-induced soil degradation. Soil in the ideal range enhances nutrient intake, lowers salinity threats, and maintains productivity year-round.

3. Safer and Healthier Produce

Farm-level pesticide/toxin strip testing gives farmers advance notice of chemical residues.It promotes secure use of pesticides and can enhance market value of produce described as "residue-safe" or "clean."Long-term adoption can allow farmers to comply with food safety requirements for export or high-value domestic markets.

4. Cost Reduction and Higher Profitability

Lower wastage of inputs: water, fertilizer, and pesticide use can be minimized.Less labor in manual monitoring (as IoT sensors make measurements automatic).Long-term increased quality of yield and input savings lead to greater net farm income.

5. Knowledge Empowerment

Farmers have access to objective information about their soil and crops instead of acting based solely on experience.This develops scientific decision-making capacity and confidence in adopting technology.The weekly visualizations/reports also enhance the sense of concealed soil health problems.

6. Alignment with Market and Policy

With more government and private markets focusing on sustainability, SafeCrop puts farmers in a position to gain from programs that incentivize water-saving measures, residue-free agriculture, and climate-smart farming.Future potential eligibility for subsidies, carbon credits, or certification schemes.

7. Long-Term Social and Environmental Returns

Groundwater exploitation decreased through efficient irrigation.Reduced chemical residues in food and soil → improved community health.Scaling adoption can make contribution to food security and sustainable agriculture in the region.

SUSTAINABILITY AND SCALABILITY

1 Environmental Sustainability

Water Conservation: By watering only when needed, the system limits groundwater overextraction and ensures effective distribution of water.Soil Conservation: Real-time soil pH and EC monitoring prevents long-term destruction and ensures soil fertility.Lower Chemical Load: Cost-effective toxin monitoring discourages overuse of pesticides/fertilizers, reducing soil and water contamination.Climate Resilience: Incorporation of weather prediction allows farmers to adjust irrigation schedules with rainfall patterns, reducing climate-related risks.

2. Economic Sustainability

Affordable Implementation: Modular design enables low-cost pilots (~₹7,500/acre) and subsequent cost savings with scale (₹5,000–₹6,000/acre).Operational Savings: The farmer saves on water, electricity/diesel for pump usage, and on fertilizers.Market Advantage: Produce certified by SafeCrop (toxin-free, residue-monitored) can command higher returns, enhancing farmers' incomes.Subsidy Support: Government irrigation and innovation programs (e.g., PMKSY, NABARD) lower farmer investment further, facilitating long-term adoption.

3. Social Sustainability

Farmer Empowerment: Data-driven insights minimize reliance on guesswork and traditional risk-prone techniques.Health & Nutrition: Safer produce benefits farm households and consumers alike.Community Impact: Cumulative deployment in villages can result in collective water savings, toxin elimination, and enhanced rural livelihoods.

4. Scalability

Modular Design: SafeCrop can be scaled acre by acre, initially with a simple sensor setup and moving up to sophisticated AI/cloud capabilities.Cluster-Level Efficiency: Collective gateways (LoRa or Wi-Fi) enable numerous farms to link up, minimizing per-acre hardware expense.Data Integration: Centralized databases enable analysis of trend patterns across regions, informing government and NGO decision-making.Interoperability: The system is capable of being integrated with available irrigation systems (drip/sprinkler) and farm apps, hence making it universally adoptablGlobal Potential: Piloted in Indian farming context, yet the method can be optimized for other developing regions that are similarly plagued by water scarcity and soil health issues.

LIMITATIONS AND CHALLENGES

Environmental Sustainability

Water Conservation: By watering only when needed, the system limits groundwater overextraction and ensures effective distribution of water.Soil Conservation: Real-time soil pH and EC monitoring prevents long-term destruction and ensures soil fertility.Lower Chemical Load: Cost-effective toxin monitoring discourages overuse of pesticides/fertilizers, reducing soil and water contamination.Climate Resilience: Incorporation of weather prediction allows farmers to adjust irrigation schedules with rainfall patterns, reducing climate-related risks.

2. Economic Sustainability

Affordable Implementation: Modular design enables low-cost pilots (~₹7,500/acre) and subsequent cost savings with scale (₹5,000–₹6,000/acre).Operational Savings: The farmer saves on water, electricity/diesel for pump usage, and on fertilizers.Market Advantage: Produce certified by SafeCrop (toxin-free, residue-monitored) can command higher returns, enhancing farmers' incomes. Subsidy Support: Government irrigation and innovation programs (e.g., PMKSY, NABARD) lower farmer investment further, facilitating long-term adoption.

3. Social Sustainability

Farmer Empowerment: Data-driven insights minimize reliance on guesswork and traditional risk-prone techniques.Health & Nutrition: Safer produce benefits farm households and consumers alike.Community Impact: Cumulative deployment in villages can result in collective water savings, toxin elimination, and enhanced rural livelihoods.

4. Scalability

Modular Design: SafeCrop can be scaled acre by acre, initially with a simple sensor setup and moving up to sophisticated AI/cloud capabilities.Cluster-Level Efficiency: Collective gateways (LoRa or Wi-Fi) enable numerous farms to link up, minimizing per-acre hardware expense.Data Integration: Centralized databases enable analysis of trend patterns across regions, informing government and NGO decision-making.Interoperability: The system is capable of being integrated with available irrigation systems (drip/sprinkler) and farm apps, hence making it universally adoptable.Global Potential: Piloted in Indian farming context, yet the method can be optimized for other developing regions that are similarly plagued by water scarcity and soil health issues.

GOVERNMENT SCHEMES AND SUPPORT

The Smart Irrigation System Based on Weather Data ,IOT ,and AI project is in direct alignment with the Indian government's agenda of doubling farmers' income, enhancing water-use efficiency, and embracing smart agriculture technologies. Multiple schemes and institutional facilities can be utilized to pilot and scale the project:

1. Irrigation & Water Management Schemes

Pradhan Mantri Krishi Sinchai Yojana (PMKSY)Emphasis: "Har Khet Ko Paani" (water to every field) and "More Crop Per Drop."Offers subsidies for drip irrigation, sprinkler irrigation, and micro-irrigation.SafeCrop can be integrated with these subsidized systems for computerized scheduling of irrigation.Atal Bhujal Yojana (ATAL JAL)Emphasizes groundwater management for water-stressed regions.Initiatives like SafeCrop that ensure proper irrigation can be aided in priority districts.

2. Digital & Smart Farming Support

Digital Agriculture Mission (2021–2025)Promotes applications of IoT, AI, and remote sensing in agriculture.SafeCrop falls under this program as a data-driven, AI-based solution.National e-Governance Plan in Agriculture (NeGPA)Supports IT-based interventions for farmers.Funds projects utilizing mobile apps, cloud platforms, and decision-support systems.

3. Innovation & Startup Funding

Rashtriya Krishi Vikas Yojana – RAFTAAROffers funding and incubation support for agri-startups.SafeCrop may be eligible for seed-stage or pilot funding under the scheme.Startup India & Atal Innovation MissionProvides grants, incubation, and credit support to innovators in agriculture technology.Incubators under AIM and NABARD may provide space for SafeCrop pilots.NABARD Agri-Tech FinancingFinances rural innovation and enables deployment of new agri-tech solutions at low interest and grants.

4. Renewable Energy & Power Subsidy Support

PM-KUSUM (Pradhan Mantri Kisan Urja Suraksha evam Utthan Mahabhiyan)Solar pumps and decentralized renewable energy subsidy.SafeCrop's IoT-based irrigation pumps can be solar-powered with subsidy of up to 60–70% of cost.

5. Research & Academic Support

ICAR (Indian Council of Agricultural Research)Appropriates finance for soil health pilot projects, water management, and precision agriculture.DST (Department of Science & Technology) and DBT (Department of Biotechnology)Offer research grant support for AI in agriculture, IoT installations, and bio-sensor technology.Summary of Government Support OpportunitiesSubsidy: PMKSY (irrigation), PM-KUSUM (solar energy).Financing: RKVY-RAFTAAR, Startup India, NABARD.Policy Alignment: Digital Agriculture Mission, NeGPA, Atal JAL.Research Grants: ICAR, DST, DBT.Smart Irrigation  System Using Weather data,IOT, and AI can therefore gain from a mixed support system: subsidies for hardware from the government, start-up financing for growing, and research funding for product development.

CONCULSION

Agriculture is the backbone of human existence, yet it remains under mounting pressure from climate change, water shortages, soil erosion, and the excessive use of chemical inputs. Small-scale farmers in particular, with limited landholdings, tend to lack the equipment and skills necessary to deal effectively with these issues. The SafeCrop project was conceived as a way to solve these issues by combining IoT sensors, meteorology, soil monitoring, and Artificial Intelligence (AI) in one easy-to-use solution for farmers.

The project shows that smart irrigation, together with monitoring of soil health and general toxin detection, can make farming both more efficient and sustainable. The project's addition to existing systems that concentrate on irrigation alone is the monitoring of soil pH, nutrient (EC), and pesticide residues, which provides farmers with a holistic picture of their farm's health. Through this data-driven methodology, farmers can more easily determine when to irrigate, how much to irrigate, and how to treat inputs like fertilizers and pesticides responsibly.

A pilot trial demonstrates the system can be created for a cost of ₹7,000–₹8,000 per acre, making it low-cost for low-income farmers as well. In addition, through government subsidies like those under PMKSY (irrigation), NABARD (agri-tech financing), and PM-KUSUM (solar energy for pumps), the effective cost can be lowered to almost half, with a consequent broader adoption. With time, the system also saves recurring expenses in terms of water, electricity, and fertilizers, enabling farmers to get their return on investment within one or two crop cycles. Its impact on farmers is manifold:Water Conservation: Optimal irrigation saves up to 30–40% of water.Improved Productivity: Harmonious soil management increases crop yield.Healthy Food Production: Monitoring of pesticide residues results in healthier crops.Improved Economy: Reduced input costs and potential for premium market prices enhance profitability.Empowerment: Farmers develop faith in scientific, data-based decision-making.

At a larger level, SafeCrop supports environmental sustainability through groundwater conservation, avoidance of soil erosion, and chemical contamination reduction. At a social level, it enhances the livelihood of farmers and health of people in their communities, and is also in line with India's vision of Digital Agriculture under initiatives like the Digital Agriculture Mission and NeGPA.

Although there are challenges in terms of sensor precision, connectivity to the internet, awareness among farmers, and training requirements, these can be addressed. With effective design enhancements, training modules, and cluster-level rollouts, SafeCrop is capable of overcoming them. More importantly, its modular and scalable design supports phased uptake beginning with small 1-acre pilots through to wider community or regional levels.

In summary, SafeCrop is not only a technological innovation but an integrated solution to contemporary agriculture. It captures the elements of sustainability, affordability, and scalability, providing a route to enable farmers to prosper in times of uncertainty. Through saving resources, enhancing productivity, and delivering safe food, the project can potentially usher in lasting advantages to farmers, consumers, and the environment. SafeCrop is thereby a move in the direction of future-proof agriculture—a model where technology and sustainability are companions to guarantee food security and farmer prosperity.

REFERENCES

Balendonck, J., Hemming, J., van Tuijl, B., Pardossi, A., Incrocci, L., & Marzialetti, P. (2014). Sensors and wireless sensor networks for irrigation management under deficit conditions (FLOW-AID Project). Agricultural Water Management, 120, 23–36. https://doi.org/10.1016/j.agwat.2012.10.014

Dhivya, S., & Manickam, R. (2020). IoT-based smart irrigation system for precision agriculture. International Journal of Scientific & Technology Research, 9(03), 1346–1350.

FAO. (2017). The future of food and agriculture – Trends and challenges. Food and Agriculture Organization of the United Nations. http://www.fao.org/publications

Gutiérrez, J., Villa-Medina, J. F., Nieto-Garibay, A., & Porta-Gándara, M. Á. (2014). Automated irrigation system using a wireless sensor network and GPRS module. IEEE Transactions on Instrumentation and Measurement, 63(1), 166–176. https://doi.org/10.1109/TIM.2013.2276487

Indian Council of Agricultural Research (ICAR). (2021). Handbook of agriculture: Principles and practices. New Delhi: ICAR Publications.

Jha, K., Doshi, A., Patel, P., & Shah, M. (2019). A comprehensive review on automation in agriculture using artificial intelligence. Artificial Intelligence in Agriculture, 2, 1–12. https://doi.org/10.1016/j.aiia.2019.05.004

Kaur, H., & Singh, G. (2019). Role of IoT in agriculture: A review of applications and challenges. International Journal of Computer Applications, 182(6), 32–37.

Ministry of Agriculture & Farmers' Welfare, Government of India. (2021). Pradhan Mantri Krishi Sinchai Yojana (PMKSY).

Ministry of New and Renewable Energy (MNRE), Government of India. (2020). Pradhan Mantri Kisan Urja Suraksha evam Utthan Mahabhiyan (

NABARD. (2022). Financing innovations and agri-tech start-ups in India. National Bank for Agriculture and Rural Development. Retrieved from: https://www.n

Singh, G., & Kaur, P. (2018). Application of IoT in agriculture: A review. International Journal of Advanced Research in Computer Science, 9(2), 773–778. Srivastava, P. K., & Shukla, R. (2020). Cloud computing and AI for smart agriculture: Future prospects. International Journal of Computer Science Trends and Technology, 8(5), 12–18.

World Bank. (2020). Water in agriculture. The World Bank Group. Retrieved from: https://www

Yadav, S., Kumar, R., & Sharma, P. (2021). IoT and AI-enabled smart farming: A step towards sustainable agriculture. Materials Today: Proceedings, 46, 9187–9191. https://doi.org/10.1016/j.matpr.2021.04.426

Zhang, Y., Wang, L., & Chen, J. (2020). A review on precision irrigation technologies for sustainable agriculture. Journal of Cleaner Production, 273, 122904. https://doi.org/10.1016/j.jclepro.2020.122904

Government of India, Department of Agriculture and Farmers' Welfare. (2022). Digital Agriculture Mission 2021-2025. https://agric

Atal Innovation Mission (AIM), NITI Aayog. (2021). Fostering startups in agriculture and rural innovation. https://

International Water Management Institute (IWMI). (2019). Water productivity and irrigation efficiency. IWMI Publications.

Patel, K., & Rathore, A. (2021). Smart irrigation systems using IoT and weather forecasting: A survey. Journal of Emerging Technologies and Innovative Research, 8(5), 144-151.

United Nations. (2019). Sustainable Development Goals Report. UN Publications. https://unstats.un